- Chinese VOS observation data

Transport format = ASCII

Naming convention = obsyear +‘_’+SHIPNAME

A file is providing the data for a given year. Example for 2019

Ship Names:

- 2019_YU KUN.TXT

- 2019_XUE LONG.TXT

- 2019_XIN YAN TIAN.TXT

- 2019_XIN XIA MEN.TXT

- 2019_XIN CHI WAN.TXT

- 2019_XIANG YANG HONG09.TXT

- 2019_HE CHEN.TXT

| 序号 | Column | Parameter | unit | P02 | P01 | SN | P06 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1-4 | YEAR |  |  |  |  |  |
| 2 | 5-8 | MONTH |  |  |  |  |  |
| 3 | 9-12 | DAY |  |  |  |  |  |
| 4 | 13-16 | HOUR（GMT） |  |  |  |  |  |
| 5 | 18-22 | ship sign |  |  |  |  |  |
| 6 | 24-28 | Latitude of observation |  |  |  |  |  |
| 7 | 30-35 | Longitude of observation |  |  |  |  |  |
| 8 | 37-39 | Wind direction | ° | EWSB | EWDAZZ01 | WDIR | DEGN |
| 9 | 42-43 | Wind speed | m/s | EWSP | EWSBZZ01 | WSPD | UVAA |
| 10 | 45-49 | Air temperature | ℃ | CDTA |  |  | UPAA |
| 11 | 51-56 | Sea level pressure | mb | CAPH |  |  | UPBB |
| 12 | 58 | QC FLAG FOR TIME | q means questionable,blank means good |  |  |  |  |
| 13 | 59 | QC FLAG FOR POSITION | q means questionable,blank means good |  |  |  |  |
| 14 | 60 | QC FLAG FOR WIND direction | q means questionable,blank means good |  |  |  |  |
| 15 | 61 | QC FLAG FOR WIND speed | q means questionable,blank means good |  |  |  |  |
| 16 | 62 | QC FLAG FOR AIR temperature | q means questionable,blank means good |  |  |  |  |
| 17 | 63 | QC FLAG FOR Sea level pressure | q means questionable,blank means good |  |  |  |  |